IMU-Based Barbell Tracker

Feature Specification & Competitive Positioning

What We Track, Why It Matters, and How We Stand Out

Darius Sattari & Mashruf Mahin

March 2026

# Why This Device Matters

Commercial velocity-based training (VBT) devices like Push Band, RepOne, and GymAware have helped bring objective measurement to strength training — but they all share the same ceiling. They measure how fast the bar moves. None of them capture the full story of how it moves. This device does.

By mounting two IMU sensors on the barbell and combining accelerometer and gyroscope data across all six degrees of freedom, this tracker computes not just velocity but bar path, asymmetry, sticking points, and fatigue patterns that no consumer tool currently delivers — at a fraction of the cost.

# Complete Feature List

Computable Now = YES  |  YES* = requires firmware development but hardware supports it  |  later = future work

| Metric / Feature | Why Coaches Care | Computable Now? | Differentiator? |
| --- | --- | --- | --- |
| CORE VELOCITY-BASED TRAINING METRICS |  |  |  |
| Mean Concentric Velocity (MCV) | The #1 VBT metric. Dictates load prescription and daily readiness. | YES | ★ |
| Peak Velocity | Explosive power output per rep. Drops faster than MCV under fatigue. | YES | ★ |
| Velocity at % 1RM | Builds athlete's personal load-velocity profile. Foundation of VBT programming. | YES | ★ |
| 1RM Estimation | Predict max from submaximal sets. No more risky max testing. | YES | ★ |
| Velocity Loss % | Autoregulate volume. Stop the set when velocity drops >20% = optimal fatigue. | YES | ★★ |
| Rep Duration | Time under tension per rep. Controls hypertrophy stimulus. | YES | ★ |
| Eccentric : Concentric Ratio | Flags technique breakdown under fatigue. Injury risk signal. | YES | ★★ |
| ADVANCED PERFORMANCE METRICS |  |  |  |
| Bar Path Trajectory (3D) | Visualize where the bar actually travels. Reveal inefficient movement patterns. | YES* | ★★ |
| Sticking Point Detection | Flags exactly where in ROM velocity drops. Tells coach which accessory work to prescribe. | YES* | ★★★ |
| Asymmetry / Bar Tilt | Left-right imbalance detection. Injury prevention. Almost no consumer tool does this well. | YES* | ★★★ |
| Minimum Velocity Threshold (MVT) | True technical failure point. More accurate 1RM prediction. | YES* | ★★ |
| Power Output (W) | Force × velocity approximation. Useful for sport-specific power profiling. | YES* | ★ |
| Bar Oscillation / Whip Frequency | Barbell flex under heavy load is measurable. Unique research signal. | YES* | ★★ |
| SET & SESSION INTELLIGENCE |  |  |  |
| Rep Counting (auto) | Hands-free rep tracking. Removes manual logging error. | YES | ★ |
| Set-to-Set Fatigue Trend | Track velocity across ALL sets, not just within one. Catch systemic fatigue. | YES | ★★ |
| Daily Readiness Indicator | Compare today's velocity at a known load vs. personal baseline. Flag under-recovery. | YES* | ★★★ |
| Rest Interval Detection | Auto-detect when rest begins/ends. Measure actual recovery between sets. | YES | ★ |
| Lift Type Classification | Auto-classify squat vs. bench vs. deadlift from motion signature. | later | ★★ |
| COACHING & ATHLETE MANAGEMENT |  |  |  |
| Athlete Profiling (load-velocity curve) | Every athlete has a unique force-velocity fingerprint. Track it over time. | YES | ★★ |
| Longitudinal Progress Tracking | Week-over-week velocity trends at same load = objective strength gain metric. | YES | ★★ |
| In-Season vs. Off-Season Benchmarking | Coaches need to know if strength is being maintained during sport season. | YES* | ★★ |
| Multi-Athlete Comparison | Compare velocity profiles across athletes at same relative intensity. | later | ★ |
| Session RPE Correlation | Correlate subjective effort rating with objective velocity data. | later | ★★ |

★★★ = major differentiator from all commercial devices  ·  ★★ = meaningful advantage  ·  ★ = table stakes (expected)

# The Three Features That Set Us Apart

These are the capabilities no commercially available VBT device handles well today. They are our strongest claims to original contribution.

## 1. Bar Tilt & Asymmetry Detection

Every strength coach eyeballs bar tilt during squats and bench press — it's one of the most common injury risk signals they watch for. But no consumer VBT device quantifies it. Our dual-IMU configuration, with sensors mounted at 90° to each other, gives us full rotational data around the bar's longitudinal axis. We can output a continuous tilt angle throughout the rep and flag asymmetry automatically.

- Catches left-right muscle imbalance before it becomes an injury

- Quantifies what coaches currently estimate by eye

- Particularly valuable for return-to-play monitoring post-injury

## 2. Sticking Point Detection

The sticking point is the portion of a lift where velocity drops the most — the moment where the athlete is most likely to fail. Identifying it precisely tells the coach exactly which muscle group or joint angle is the limiting factor, which directly informs accessory work programming. Current VBT devices give you a velocity curve but none automatically identify or annotate the sticking point.

- Squat: typically the hole (bottom) or mid-ascent hip shift

- Bench: typically 2–3 inches off the chest or lockout

- Deadlift: typically just below the knee

- Output: position as % of range of motion, velocity at that point, trend over time

## 3. Daily Readiness Indicator

This is arguably the most practically useful feature for a strength coach managing a team. The concept: every athlete, at a known submaximal load, will produce a characteristic velocity. On a well-recovered day that velocity is higher; on a fatigued or under-recovered day it is lower. By comparing today's velocity at a warm-up set weight to the athlete's personal historical average at that load, we can flag readiness before training begins.

- A 5% velocity drop at warm-up load correlates with poor recovery state

- Enables autoregulated programming — reduce volume before the athlete over-trains

- Requires longitudinal data collection to build baseline — this is a key reason to start collecting early

# Competitive Comparison

How our device stacks up against the three leading commercial VBT tools on the market:

| Feature | Push Band | RepOne | GymAware | Your Device |
| --- | --- | --- | --- | --- |
| Mean Concentric Velocity | ✓ | ✓ | ✓ | ✓ |
| 1RM Estimation | ✓ | ✓ | ✓ | ✓ |
| Velocity Loss % | ✓ | ✓ | ✓ | ✓ |
| Rep Counting | ✓ | ✓ | ✓ | ✓ |
| 3D Bar Path Reconstruction | ✗ | ✗ | ~ | ✓ |
| Bar Tilt / Asymmetry Detection | ✗ | ✗ | ✗ | ✓ |
| Sticking Point Detection | ✗ | ✗ | ✗ | ✓ |
| Daily Readiness Indicator | ~ | ~ | ~ | ✓ |
| Open Data Access (CSV) | ✗ | ✗ | ✗ | ✓ |
| Cost | $300 | $600 | $2,500+ | ~$86 |

Note: Commercial device feature assessments based on published specifications and peer-reviewed evaluations as of 2025.

# Hardware Overview

All metrics above are derived from a ~$86 sensor package. Full hardware specifications:

| Component | Role | Spec |
| --- | --- | --- |
| ESP32-S3 Feather | Microcontroller + BLE + Wi-Fi | 240MHz dual-core, BLE 5.0 |
| 2× LSM6DSOX IMU | Motion sensing (accel + gyro) | ±16g, ±2000dps, up to 6.7kHz ODR |
| Adalogger FeatherWing | SD logging + RTC timestamps | MicroSD SPI + PCF8523 RTC |
| 8GB MicroSD | Data storage | FAT32, holds 9+ days at 200Hz |
| 500mAh LiPo | Portable power | 2–3 hrs runtime, USB-C charging |
| IP65 ABS Enclosure | Protection + mounting | 63×58×35mm, weatherproof |
| 2″ P-Clamp + Foam Tape | Barbell attachment | Fits 50mm Olympic sleeve |

What the Dual-IMU Configuration Unlocks - IMU #1 (X-axis along bar length): captures primary concentric/eccentric bar travel - IMU #2 (rotated 90°): captures bar rotation, tilt, and lateral deviation - Together: full 6-DOF motion data, enabling 3D bar path, asymmetry, and rotation metrics - Cross-validation between sensors reduces integration drift in velocity computation

# What NCAA Strength & Conditioning Coaches Need

Based on NCAA Division I strength and conditioning workflows, these are the outputs coaches use day-to-day and the decisions they drive:

## Daily Programming Decisions

- Did the athlete recover? → Daily Readiness Indicator

- How much volume today? → Velocity Loss % to autoregulate set termination

- Are they getting stronger? → Week-over-week MCV trend at same relative load

## Technique & Injury Prevention

- Is the bar path deviating under fatigue? → 3D bar path reconstruction

- Is the athlete shifting left or right? → Asymmetry / tilt detection

- Where is the sticking point and is it getting worse? → Sticking point detection

- Are they rushing the eccentric? → Eccentric : concentric time ratio

## Load Management & Periodization

- What load produces peak power for this athlete? → Load-velocity profiling

- Is the athlete maintaining strength during the sport season? → In-season benchmarking

- How close is the athlete to their true max without testing it? → 1RM estimation via MVT

## Reporting & Compliance

- Open CSV export means data plugs into any existing team analytics pipeline

- Per-session and per-athlete logs usable in compliance reporting

- Timestamped sessions with RTC enable longitudinal comparison across months and seasons

# Original Research Contribution

The academic framing that distinguishes this project from prior work:

The Gap We Fill - No existing public dataset combines barbell-mounted IMU data with ground-truth velocity across all three powerlifts (squat, bench, deadlift) - No consumer VBT device quantifies bar tilt or sticking point automatically - No open-source barbell tracker uses the LSM6DSOX with dual-IMU daisy-chaining for 3D reconstruction - OpenCap validation gives our dataset a ground-truth anchor that most IMU studies lack

What We Will Produce - A novel dataset: dual-IMU barbell motion data + OpenCap ground truth, across squat / bench / deadlift - A validated pipeline for 1RM estimation, rep counting, and bar path reconstruction under $100 - Quantitative evidence for or against dual-IMU advantage over single-IMU for each metric - Codebase and hardware design that other researchers can replicate