IMU Barbell Tracker

Complete Assembly Guide

ESP32-S3 Feather  +  LSM6DSOX IMU  +  Adalogger FeatherWing

Recommended $86 Build

Darius Sattari & Mashruf Mahin

March 2026

# Before You Begin: Tools & Supplies

Before starting assembly, gather these tools. Nothing here requires soldering — the entire build uses plug-and-play connectors and stacking headers.

- Small Phillips-head screwdriver (for P-clamp bolts)

- Drill with 5mm and 8mm bits (for enclosure holes)

- Hobby knife or deburring tool

- Double-sided foam tape (thin, ~2mm)

- M4 bolt + nut (2x) — from any hardware store, ~$0.50

- USB-C cable (for programming and charging)

- Computer with Arduino IDE or CircuitPython installed

Zero Soldering Required Every electrical connection in this build uses either STEMMA QT click-connectors or stacking pin headers. If your ESP32-S3 Feather came with loose headers, those do need to be soldered on — but many units ship with headers pre-soldered. Check your package.

# Phase 1: Parts Inventory

Lay out all your parts and check them against this list. Mark each item as you confirm it.

| # | Component | Product | Have it? |
| --- | --- | --- | --- |
| 1 | IMU Sensor | Adafruit LSM6DSOX (#4438) |  |
| 2 | Microcontroller | ESP32-S3 Feather (#5477) |  |
| 3 | Data Logger Wing | Adalogger FeatherWing (#2922) |  |
| 4 | MicroSD Card | SanDisk 16GB Ultra microSDHC |  |
| 5 | Battery | Adafruit 500mAh LiPo (#1578) |  |
| 6 | I2C Cable (build) | STEMMA QT 50mm (#4399) |  |
| 7 | I2C Cable (proto) | STEMMA QT to headers 150mm (#4209) |  |
| 8 | Mounting Clamps | ISPINNER 2" P-clamps (12-pack) |  |
| 9 | Enclosure | Zulkit IP65 ABS box 63x58x35mm |  |
| 10 | Backup Straps | 2" cinch straps (5-pack) |  |
| 11 | Jumper Wires | ELEGOO 120-piece Dupont set |  |

Battery Polarity Warning If you substituted an Amazon LiPo battery (e.g., EEMB), verify the JST-PH connector polarity matches Adafruit’s convention BEFORE plugging it in. Adafruit uses positive on the right when the connector tab faces you. Reversed polarity will permanently destroy the ESP32-S3 Feather.

# Phase 2: Electronics Assembly

This phase connects all electronic components. The Feather ecosystem makes this remarkably simple — most connections are just plugging things together.

## Connection Overview

| From | Pin/Port | To | Pin/Port |
| --- | --- | --- | --- |
| ESP32-S3 Feather | STEMMA QT port | LSM6DSOX | STEMMA QT port |
| ESP32-S3 Feather | Stacking headers | Adalogger Wing | Stacking headers |
| Adalogger Wing | MicroSD slot | SanDisk 16GB | (inserted) |
| ESP32-S3 Feather | JST-PH battery | 500mAh LiPo | JST-PH connector |

Step 1: Stack the Adalogger FeatherWing onto the ESP32-S3 Feather

The Adalogger FeatherWing is designed to stack directly on top of the ESP32-S3 Feather using pin headers. Align the pins carefully — the FeatherWing’s female headers should slide onto the Feather’s male header pins. The boards should sit parallel with about 8mm of clearance between them.

- The Adalogger’s MicroSD slot should face outward for easy card access

- Press firmly but evenly — don’t force it at an angle

- The SPI pins (SCK, MOSI, MISO, and a chip-select pin) connect automatically through the stacking headers

Pin Assignment Note The Adalogger FeatherWing uses SPI for the MicroSD card. On the ESP32-S3 Feather, the default SPI pins are: SCK = GPIO36, MOSI = GPIO35, MISO = GPIO37. The SD chip select defaults to GPIO5. The RTC communicates over I2C (same bus as your IMU) at address 0x68.

Step 2: Connect the LSM6DSOX IMU via STEMMA QT

Take the 50mm STEMMA QT cable (#4399) and click one end into the STEMMA QT port on the ESP32-S3 Feather (it’s the small 4-pin connector on the board edge). Click the other end into the STEMMA QT port on the LSM6DSOX breakout board.

That’s it — the I2C connection (SDA, SCL, 3.3V, GND) is now complete. The cable is keyed so it can only go in one way.

- The LSM6DSOX default I2C address is 0x6A (with the SDO/SA0 pin pulled low)

- If you later add a second IMU, cut the address jumper on one board to change it to 0x6B

- The RTC on the Adalogger also uses I2C (address 0x68) — no conflict, they share the bus

Cable Length Matters Use the 50mm cable for the final build (keeps the IMU close to the microcontroller inside the enclosure). Use the 150mm STEMMA QT-to-headers cable (#4209) only during prototyping and testing on a breadboard.

Step 3: Insert the MicroSD Card

Insert the SanDisk 16GB microSD card into the Adalogger FeatherWing’s MicroSD slot. The card clicks in with a push-push mechanism (push to insert, push again to eject).

Before first use, format the card as FAT32 using your computer. The ESP32-S3 works best with FAT32-formatted cards under 32GB.

Step 4: Connect the LiPo Battery

Plug the 500mAh LiPo battery’s JST-PH connector into the battery port on the ESP32-S3 Feather (the 2-pin white connector). The connector is keyed and should only fit one way.

- The Feather’s built-in MAX17048 fuel gauge will automatically track battery level

- Charging happens automatically via USB-C when plugged in (500mA charge rate)

- Expected runtime: 2–3 hours continuous (adequate for a 60–90 min training session)

Battery Placement For the final enclosure build, the battery will sit beneath or beside the stacked Feather boards. The 500mAh cell measures 29 x 36 x 4.75mm and weighs only 10.5g. Use a small piece of foam tape to secure it inside the enclosure so it doesn’t shift during lifts.

At this point, your electronics stack should look like this (bottom to top): ESP32-S3 Feather → Adalogger FeatherWing (stacked on top) → STEMMA QT cable running to the LSM6DSOX board → LiPo battery plugged into the Feather’s JST port.

# Phase 3: Initial Power-Up & Verification

Before putting everything in the enclosure, verify that all components work. This saves you from having to disassemble later.

Step 1: Install Arduino IDE and Board Support

- Install Arduino IDE 2.x from arduino.cc

- In Arduino IDE, go to File → Preferences → Additional Board Manager URLs

- Add: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Go to Tools → Board → Board Manager, search for "esp32", install the Espressif package

- Select board: "Adafruit Feather ESP32-S3" from the ESP32 section

Step 2: Install Required Libraries

In Arduino IDE, go to Sketch → Include Library → Manage Libraries, and install:

- "Adafruit LSM6DS" by Adafruit (includes LSM6DSOX support)

- "Adafruit Unified Sensor" by Adafruit (dependency)

- "SD" (built-in, should already be present)

- "RTClib" by Adafruit (for the Adalogger’s PCF8523 RTC)

Step 3: Upload the Hardware Verification Sketch

Connect the ESP32-S3 Feather to your computer via USB-C. Select the correct COM port in Arduino IDE (Tools → Port). Copy and upload the following test sketch:

// IMU Barbell Tracker — Hardware Verification Sketch

// Tests: IMU (I2C), SD card (SPI), RTC (I2C), Battery gauge

#include <Adafruit_LSM6DSOX.h>

#include <SD.h>

#include <RTClib.h>

#include <Wire.h>

Adafruit_LSM6DSOX lsm;

RTC_PCF8523 rtc;

const int SD_CS = 5; // Adalogger default chip select

void setup() {

Serial.begin(115200);

while (!Serial) delay(10);

delay(1000);

Serial.println("=== IMU Barbell Tracker: Hardware Test ===");

// Test 1: IMU

Wire.begin();

if (lsm.begin_I2C()) {

Serial.println("[PASS] LSM6DSOX found at 0x6A");

lsm.setAccelRange(LSM6DS_ACCEL_RANGE_16_G);

lsm.setGyroRange(LSM6DS_GYRO_RANGE_2000_DPS);

lsm.setAccelDataRate(LSM6DS_RATE_416_HZ);

lsm.setGyroDataRate(LSM6DS_RATE_416_HZ);

Serial.println(" Config: 16g, 2000dps, 416Hz");

} else {

Serial.println("[FAIL] LSM6DSOX not found!");

}

// Test 2: RTC

if (rtc.begin()) {

Serial.println("[PASS] PCF8523 RTC found");

if (!rtc.initialized() ¦¦ rtc.lostPower()) {

rtc.adjust(DateTime(F(__DATE__), F(__TIME__)));

Serial.println(" RTC set to compile time");

}

} else {

Serial.println("[FAIL] PCF8523 RTC not found!");

}

// Test 3: SD Card

if (SD.begin(SD_CS)) {

Serial.println("[PASS] MicroSD card detected");

File f = SD.open("/test.txt", FILE_WRITE);

if (f) {

f.println("IMU Tracker SD test");

f.close();

Serial.println(" Write test: OK");

}

} else {

Serial.println("[FAIL] MicroSD not detected!");

}

Serial.println("\n=== Starting live IMU stream (5 sec) ===");

}

void loop() {

sensors_event_t accel, gyro, temp;

lsm.getEvent(&accel, &gyro, &temp);

Serial.printf("Accel: %+7.2f %+7.2f %+7.2f ",

accel.acceleration.x,

accel.acceleration.y,

accel.acceleration.z);

Serial.printf("Gyro: %+8.2f %+8.2f %+8.2f\n",

gyro.gyro.x, gyro.gyro.y, gyro.gyro.z);

delay(100); // 10 Hz for readability

}

Step 4: Read the Test Output

Open the Serial Monitor (Tools → Serial Monitor, 115200 baud). You should see:

- [PASS] for all three components (IMU, RTC, SD)

- Live accelerometer and gyroscope readings streaming at 10Hz

- With the board sitting still on a table, accel Z should read ~9.8 m/s² (gravity), and X/Y near 0

- Gyro values should be near zero when stationary (small noise is normal)

If the IMU Test Fails Double-check the STEMMA QT cable is fully seated on both ends (you should hear/feel a click). If using the 150mm prototyping cable with male headers on a breadboard, ensure SDA goes to SDA and SCL goes to SCL. The I2C address scan might show 0x6B instead of 0x6A if the address jumper was cut — update the code accordingly.

# Phase 4: Enclosure Preparation

The Zulkit IP65 enclosure protects the electronics during heavy lifting. The internal dimensions (63 x 58 x 35mm) are a snug fit for the stacked boards + battery, which is ideal — minimal movement inside means less vibration-induced noise in your IMU data.

Step 1: Plan Your Hole Layout

You need to drill two or three holes in the enclosure:

- USB-C access port (one side wall): ~8 x 3mm rectangular opening, or a 10mm round hole that fits the USB-C plug

- P-clamp mounting bolt hole (bottom): 5mm diameter, centered along the long axis of the bottom face

- Optional second bolt hole: 5mm, ~35mm from the first, for a two-bolt P-clamp mount (recommended for extra rigidity)

Drilling ABS Plastic ABS drills easily but can crack if you push too hard. Start with a small pilot hole (2mm), then step up to your final size. Go slow, let the bit do the work. For the USB-C slot, drill a 3mm hole and widen it with a hobby knife or small file.

Step 2: Line the Interior with Foam Tape

Cut thin strips of double-sided foam tape and apply them to the bottom interior of the enclosure. This serves two purposes: it cushions the electronics against impact shock, and it immobilizes the boards so they don’t rattle. Leave the top adhesive backing in place for now — you’ll press the electronics stack onto it once everything is tested.

Step 3: Dry-Fit the Electronics

Before committing, place the electronics stack inside the enclosure (without removing the foam tape backing) to verify:

- The USB-C port on the Feather aligns with your drilled hole

- The MicroSD card slot on the Adalogger is still accessible (even if you need tweezers)

- The IMU board and STEMMA QT cable fit without sharp bends

- The battery fits in the remaining space

- The lid closes fully

IMU Orientation Matters Pay attention to which axis of the LSM6DSOX points along the barbell. Ideally, mount the IMU board so its X-axis (marked on the silkscreen) aligns with the bar’s longitudinal axis. Document which axis points which direction — you’ll need this mapping in firmware for gravity compensation and bar path reconstruction.

# Phase 5: Barbell Mounting (No 3D Printing Required)

Since you don’t have makerspace access yet, this section covers a mounting approach using only the off-the-shelf parts from your BOM. It’s robust enough for heavy powerlifting and can be upgraded later with a 3D-printed bracket.

## Method: P-Clamp + Enclosure + Cinch Strap

Step 1: Attach the P-Clamp to the Enclosure

Take one ISPINNER 2″ rubber-cushioned P-clamp and position it against the bottom of the Zulkit enclosure. The P-clamp’s mounting tab should align with the bolt hole(s) you drilled. Secure with an M4 bolt and nut. Tighten firmly but don’t over-torque (ABS can crack). If using two bolt holes, attach two P-clamps side by side for maximum rigidity.

Step 2: Clamp onto the Barbell Sleeve

Open the P-clamp and position it around the 50mm Olympic barbell sleeve (the rotating outer portion where plates load). The rubber lining grips the smooth chrome surface. Tighten the P-clamp screw until the enclosure is snug and doesn’t rotate or slide when you twist the bar.

- Mount near the inside collar (close to the knurling) to minimize the moment arm from plate loading

- Ensure the device sits between the collar and your weight plates, so plates don’t contact the enclosure

- Mount on the same end every session for consistent data

Step 3: Add the Cinch Strap as Backup Retention

Wrap a 2″ heavy-duty cinch strap around the entire assembly (barbell sleeve + P-clamp + enclosure). This is your safety net. Even if the P-clamp loosens under extreme loads, the cinch strap keeps everything on the bar. The straps are rated for 103 lb shear strength — more than sufficient.

Alternative Mounting Idea For a one-motion attach/detach solution, buy a cheap pair of lockjaw-style Olympic barbell collars ($8–12 on Amazon) and epoxy or bolt the Zulkit enclosure directly to one collar. You then clamp the tracker on and off the bar the same way you change weight collars — this is what the commercial FORM Lifting Collar product does.

## Final Enclosure Assembly

Once mounting is verified and tested:

- Remove the foam tape backing inside the enclosure

- Press the electronics stack firmly onto the foam tape, USB-C port aligned with the access hole

- Tuck the LiPo battery into the remaining space, secured with its own foam tape strip

- Route the STEMMA QT cable so it doesn’t pinch when the lid closes

- Close the lid and verify the four screws seat properly (IP65 seal)

# Phase 6: Post-Assembly Checklist

Run through this checklist to confirm everything is working before your first data collection session.

- Power on: Press the Feather’s reset button. The onboard NeoPixel should light briefly.

- IMU test: Tilt the enclosure in all directions. If connected to Serial Monitor, accel values should change smoothly.

- SD write test: Confirm the test sketch created /test.txt on the SD card. Remove the card, check on your computer.

- RTC test: Unplug USB, wait 10 seconds, replug. The RTC should still report the correct time (not reset to 0).

- Battery test: Unplug USB, verify the board stays powered from the LiPo. Let it run for 5 minutes to confirm stability.

- Mount test: Attach to the barbell, load some weight, and perform a few gentle reps. Check the enclosure hasn’t shifted.

- Vibration test: Rack and unrack the bar firmly. Nothing should rattle or come loose.

## Next Steps

With hardware assembled and verified, you’re ready to move on to firmware development. The key milestones ahead:

- Write a high-speed data logging sketch (416 Hz, timestamped CSV to SD card)

- Implement Madgwick filter for orientation estimation

- Add Zero Velocity Update (ZVU) detection for drift correction

- Integrate gravity compensation and velocity computation

- Set up OpenCap for ground-truth validation

- Begin data collection across squat, bench press, and deadlift

Total assembly time for the hardware: approximately 1–2 hours (most of which is enclosure drilling and dry-fitting). The electronics connection itself takes under 5 minutes thanks to the STEMMA QT ecosystem.